MAKERERE UNIVERSITY

SCHOOL OF COMPUTING AND INFORMATICS TECHNOLOGY

CENTRE FOR INNOVATIONS & PROFFESSIONAL SKILLS

DEVELOPMENT

END OF RECESS SEMESTER EXAMINATION 2018/2019

PROGRAMME: BSc CSC, BIST

YEAR OF STUDY: 1

COURSE NAME: CISCO CERTIFIED NETWORK ASSOCIATE

COURSE CODE: CSC 1303

DATE: 24TH JULY 2019               TIME: 8:00AM – 11:00AM

MARKING GUIDE

SECTION A (20 MARKS). Each point is one mark = 2020

| - A | 6. C | 11. A | 16. B |
| --- | --- | --- | --- |
| - B | 7. B | 12. A |  |
| - C | 8. A | 13. A , B, C |  |
| - C | 9. D | 14. B |  |
| - C | 10. B | 15. A, E, F |  |

SECTION B

QUESTION 1 (Compulsory question)

- Consider IPv6 address:  FF02:0DB8:000A:0000:0000:CAFÉ:0000:0005/64.

  - Multicast IPv6 Address 					(02 marks)

  - FF02:DB8:A:0:0:CAFÉ:0:5/64  (any Two correct one  = 02 marks)

FF02:DB8:A::CAFÉ:0:5/64

FF02:DB8:A:0:0:CAFÉ::5/64

  - Organization may prefer using IPv6 for the following reasons:  (Any two correct one award 04 marks)

    - Increased address space in IPv6

    - Integrated security is strengthen in IPv6

    - IPv6 eliminates the need for Network Address Translation (NAT).

- Using  IPv4 address 192.168.33.0/24 to subnet Kampala, Jinja and Arua town networks is shown in this table:

  - DOG&CAT LTD IPv4 address Plan: (02 marks @ for Kla, Jinja, Arua and WAN link = 08 marks)

| Hosts/Location | Network IP address | Usable IPv4 Address Range | Broadcast IPv4 Address |
| --- | --- | --- | --- |
| 55 hosts (Kampala) | 192.168.33.0/26 | 192.168.33.1 – 192.168.33.62 | 192.168.33.63 |
| 15 Hosts (Jinja) | 192.168.33.64/27 | 192.168.33.65 – 192.168.33.94 | 192.168.33.95 |
| 10 Hosts (Arua) | 192.168.33.96/28 | 192.168.33.97 – 192.168.33.110 | 192.168.33.111 |
| WAN 1 Link | 192.168.33.112/30 | 192.168.33.113 – 192.168.33.114 | 192.168.33.115 |
| WAN 2 Link | 192.168.33.116/30 | 192.168.33.117 – 192.168.33.118 | 192.168.33.119 |

  - The sketch topology of the DOG&CAT Company Ltd network.  (01 mark for the diagram and  101 mark for IPv4 address labeled = 02 marks)

QUESTION 2

- (i) The differences between a hub and a switch are:  (02 marks for any one correct difference)

  - Hub operates at physical layer while a switch operates at data link layer of the OSI.

  - Hub has one collision domain while each port on a switch is a collision domain

  - A switch is an intelligent device while a hub is a non-intelligent device.

(ii) The differences between a Byte and a Bit are: (02 marks for any one correct difference)

- A byte consists of eight bits while a bit is consists of 0’s and 1’s.

- Byte is the unit that measures the storage capacity of data while a bit is a unit that measures the speed of data being transmitted.

- A byte represents one character of on the keyboard while a bit is the binary digit.

- (i) HTTP operates at destination port 80. 				 		 (01 mark)

(ii) Telnet operates at destination port 23.					 (01 mark)

(iii) SMTP operates at destination port 25. 					 (01 mark)

- 01001111 binary is equivalent to 4F Hexadecimal. 				 (02 marks)

- If the IPv4 address is : 192.168.20.2/27, then the subnet-mask is:

11111111.11111111.11111111.11100000 = 255.255.255.224  			 (02 marks)

- The three features of Linksys Router incorporates: (03 marks for any three correct answers)

  - A switch: which can forward frames among hosts based on MAC addresses.

  - Router: which can be used to create more than one networks.

  - Wireless Access Point: Ability to connect wireless devices.

- Addresses associated to:

  - Unicast MAC address is 00-07-E9-42-AC-28. NOTE: For Unicast the MAC address is neither Multicast nor Broadcast MAC addresses. 			(02 marks)

  - Multicast MAC address is 01-00-5E-00-00-01. NOTE: the MAC address MUST have special value that begins with 01-00-5E.  				(02 marks)

  - Broadcast MAC address is FF-FF-FF-FF-FF-FF 			 (02 marks)

QUESTION 3

- Requirements:

  - Terminating equipment (Crippling tools).

  - RJ 45.

  - UTP Cable.

  - Testing Machine.

Steps to make a crossover cable are:

- Use crippling tool to cut coated part of the UTP cable of about 2 cm length long.

(02 marks)

- Arrange the eight wires following the colour codes of: Green/white, Green, Orange/white, Blue, Blue/white, Orange, Brown/white and Brown for pin 1 to 8 respectively using standard T568A on side A of the UTP cable.                  (02 marks)

- Fix the Side A of the UTP cable arranged in RJ 45. Cripple it while maintain the colour codes arrangement. 					                 (02 marks)

- Repeat steps 1 to 3 on side B of the UTP cable using standard T568B with colour codes arrangement of Orange/white, Orange, Green/white, Blue, Blue/white, Brown/white and Brown for pin 1 to 8 respectively. 		                (02 marks)

- Finally use a testing machine to check if it’s a working Ethernet cable by confirming that pin 1 lights with pin 3 and pin 2 lights with pin 6 simultaneously.    (02 marks)

- The names of the Ethernet cables are:

  - Crossover cable 								(01 mark)

  - Crossover cable 								(01 mark)

  - Crossover cable 								(01 mark)

  - Straight through cable 							(01 mark)

- Two measures to mitigate attacks on wireless devices in your LAN are:	 (02 marks)

  - Disables SSID broadcast.

  - Change the default values such as username, passwords and IP addresses.

  - Configure encryption using WEP or WPA.

  - Use MAC address filtering system.

- The differences between single mode and multi-mode fiber optics  cables are: 	(04 marks)

| Single Mode Fiber optics. | Multimode fiber optics. |
| --- | --- |
| - Small Core. | - Larger core than Single mode. |
| - Less dispersion of light pulses. | - Allows greater dispersion and therefore, loss of signal. |
| - Suitable for long distance application. | - Suitable for long distance application but shorter than single mode. |
| - Use lasers as light source. | - Uses LEDs as the light source. |
| - Commonly used with campus backbones for distances of several thousand meters. | - Commonly used with LANs or distances of a couple hundred meters within a campus network. |

QUESTION 4

- From the figure:

  - The Next Hop IPv4 address is 192.168.10.233. 				(02 marks)

  - C = directly connected network to the Router. 				(02 marks)

  - S = the remote route to the destination was advertised by Static Route.	 (02 marks)

  - S* = the remote route to the destination was advertised using default Route (Special Static Route). 								(02 marks)

  - The command is: Router# show ip route  				(02 marks)

- Benefits of Network Address Translation (NAT) are: (Any 3 x 2 correct answers = 06 marks)

  - Use to conserve public IPv4 addresses.

  - Provides network security.

  - Provides consistency for internal networking schemes.

  - Increased flexibility of connection to public network especially when shifting from one ISP to another.

- Wild card Masks for the following IPv4 addresses are:

  - 192.168.30.0/24 is 	=0.0.0.255				(01 mark)

  - 10.10.1.2 is		 =0.0.0.0				(01 mark)

  - 172.17.3.32/27 is	 	=0.0.0.31				(01 mark)

  - 0.0.0.0 is 			=255.255.255.255 			(01 mark)

QUESTION 5

- Advantages of implementing VLANs are: (Any 3 x 2 correct answers = 06 marks)

  - Provides network security in an organizational network.

  - Shrink broadcast domains.

  - Cost reduction in terms of creating very many LANs in an organization.

  - Better performance.

  - Improved IT staff efficiency.

  - Simpler project and application management.

- The prompts on CLI of a CISCO Router indicates the following modes:

  - Router>     is User EXEC Mode.   					(01 mark)

  - Router(config)#     is the Global Configuration Mode.   		(01 mark)

  - Router# is the Privileged EXEC Mode. 				(01 mark)

  - Router(config-if)#  is the Interface Configuration Mode.   		 (01 mark)

- Router# copy run start is used to save the current configuration.		 (02 marks)

- The command Router(Config)# router ospf 20 is used to enable ospf with process ID 20 on a Router. 										(02 marks)

- The summarize route are: 								(06 marks)

10.0.0.0 = 00001010.00000000.00000000.00000000

10.0.1.0 = 00001010.00000000.00000001.00000000

10.0.2.0 = 00001010.00000000.00000010.00000000

10.0.3.0 = 00001010.00000000.00000011.00000000

11111111.11111111.11111100.00000000 = /22

Thus the summarized route is 10.0.0.0/22